[image: image3.wmf]

                                           



      CPB911

[image: image1.png]



	CPB911 – 3.5` AMD Ryzen Embedded V1404I\ Embedded R1305G based SBC

	


Embedded 3.5” Single-Board Computer for High-Performance and Low-Power Embedded Applications
technical specification
· Processor:  AMD Ryzen Embedded V1404I (CPB911-01); AMD Ryzen Embedded R1305G (CPB911-02).

· Memory: DDR4-2400 SDRAM with ECC support, soldered: 16 GB dual-channel (CPB911-01); 8 GB single-channel (CPB911-02).

· Video Output: 2x DisplayPort (up to 4K); 2x LVDS for CPB911-01 / 1x LVDS for CPB911-02, up to 1920x1200 @ 60 Hz, 18/24 bit.

· LPC Bus: Available on expansion connector; compliant with LPC Specification v1.1.

· Flash BIOS: 8 MB SPI Flash.

· MiniPCI Express: 3 sockets (2 for CPB911-02); PCIe Rev. 2.0 and USB 2.0 support; GSM modem and SIM support.

· SATA III: 2 interfaces; up to 6 Gbps.

· M.2 NVMe: Key M socket; supports 2242 / 2260 / 2280.

· Ethernet: 2x 10/100/1000/2500 Mbps controllers.

· USB: USB 1.1 / 2.0 / 3.0 support; 2x USB 3.0 Type-A.

· COM1: RS-232; up to 230 kbps; TX, RX, RTS, DTR.

· FRAM: 4 KB user data storage.

· RTC: Powered by CR2032 lithium battery.

· Operating Temperature: -40°C to +70°C (CPB911-01); -40°C to +85°C (CPB911-02).

· Power Supply: +12 V ±5%.

· OS Compatibility: FreeDOS, Windows 10, Linux.

Block Diagram
[image: image2.png]A
I

I
XP4 Tonbko CPB911-01
! HDA
XS9 m Lvbst ) ( Lvbs2
I
I
I
DP to 1 DP to
LVDS I
xs12 ! Soldered -
********* DDR4 ECC
DisplayPort 1 SDRAM SR
DisplayPort 2 up To 16GB
USB3.0 USB 3.0
e port12 Hﬂﬂ
Gigabit L
p Ethernet - A=¥E RS2:2
xpis( ZEaat 1225 AMD Ryzen Tranceiver
Embedded
Gigabit soc
Ethernet
" i225
Xs10 —
®
1x4 PCle BiES
64Mbit
SA
XP2 ( portt
xp1 ( SATA
Port0

|Tonbko CPB911-01!




Ordering information
	№
	Configurations
	Description

	1
	CPB910-01
	AMD Ryzen Embedded V1404I / 16 GB ECC DDR4 / 2x SATA / 2x Ethernet / 3x MiniPCIe / 2x LVDS

	2
	CPB910-02
	AMD Ryzen Embedded R1305G / 8 GB ECC DDR4 / 2x SATA / 2x Ethernet / 2x MiniPCIe / 1x LVDS

	3
	\Coating
	 Coating Optional 


Delivery checklist:
1. CPU  CPB911
2. Passport
3. Mounting kit
4. Package.
v 1.3                                           


©2026 Fastwel. Tel.: 7 (495) 234-0639. Fax: 7 (495) 232-1654. http://www.fastwel.com

[image: image3.wmf]